# EMBER Wildland Fire Emissions Estimates Technical Documentation

Prepared by James Beidler ORD/CEMM/AESMD/ESAB beidler.james@epa.gov on 3/14/25

DRAFT

This document is not intended for distribution outside of the specific use as reference material for the EMBER project. The views expressed in this document do not necessarily reflect the views or policies of the US EPA. This is a reference document. Nothing in this document should be considered technical direction on the part of the author under any contract or work order.

# Overview

Document Description

This document provides technical guidance for the generation of model ready wildland fire emissions using the methods developed for the Expedited Modeling of Burn Events Results (EMBER) 1. Prerequisites for the application of this guidance include the installation of the BlueSky Pipeline 2 wildland fire modeling system (https://github.com/pnwairfire/bluesky), the Python-based implementation of SmartFire 2 3, 4 (https://github.com/jlbeidler/sf2py), and the Sparse Matrix Operator Kernel Emissions model (https://github.com/CEMPD/SMOKE). This is a technical document that assumes familiarity with the use of Linux filesystems and scripting, Python, emissions models, and geospatial data processing methods. The associated package of scripts and data is provided as-is with no modification to paths, dates, or filenames. Additional input files, ancillary files, and libraries may be required to replicate the EMBER 2023 inventories. The “YYYY” notation may be used to generically specify the use of a four-digit year in a file or variable name.

Acronyms

| Acronym | Definition |
| --- | --- |
| BSP | BlueSky Pipeline |
| EMBER | Expedited Modeling of Burn Events Results |
| FCCS | Fuel Characteristic Classification System |
| FF10 | Flat file 2010 |
| HMS | Hazard Mapping System |
| JSON | JavaScript Object Notation |
| NBAC | National Burned Area Composite |
| NEI | National Emissions Inventory |
| SERA | Smoke Emissions Reference Application |
| SF2PY | SmartFire 2 Python |
| SMOKE | Sparse Matrix Operator Kernel Emissions |
| WFIGS | Wildland Fire Interagency Geospatial Services |

System Requirements

This processing stream requires a Linux system with a recent version of Postgres and Postgis, Python 3.9 or later, GDAL, SMOKE 4.9 or later, BSP 4.2.14, and multiple Python modules available from PyPi/pip.

# Preparing Wildland Fire Activity

HMS Satellite Data

HMS 5 satellite text-based fire detections data are used to spatially and temporally apportion associated wildland fire perimeters. Additionally, the HMS data provide activity where no other data sources exist. The HMS detects are pre-processed to differentiate detections over wildlands versus croplands and the US versus Canada. All HMS-related scripts are provided in the package under the activity/hms path.

A script, gethms.py, is supplied to fetch daily point files from the NOAA ftp to the system. The year and outpath variables should be adjusted as necessary to match the target year and the system output path for the daily HMS files.

The next step in the HMS process is to subset the satellite detects to those north of 24.5° latitude and associate a county or province FIPS code with each detect. This is done with the hms_set_fips.py script. Within this script the path to the northamerica_county_province_state_latlon.shp file (supplied under the geospatial directory) will need to be updated to match the location on the system path. A wrapper script, gen_hms.csh, can be used to set the command line arguments including the input path (inpath) to the HMS daily files, the writable output path (outpath) for the daily script output files, and the target year.

Once all dates have been run through hms_set_fips.py the satellite data is split into files based on country and landcover. For Canada the script hms_canada_cropsplit.py uses a combined wildland and cropland raster to associate landcover with each detect and split into a crop type (labeled hms_YYYY_canada_crops.csv) and wildland type file (labeled hms_YYYY_wildland.csv). The script should be updated to reflect the location of the fbp_2011_250m_canada_fccsmap_crops_fix.nc file supplied in the geospatial folder of the package. Similarly, the hms_fccs.py script is used to split US detects based on landcover. This script should be updated to reflect the location of the lf_220_2022_cdl_2022_120m_fix.nc raster file supplied in the geospatial folder. Outputs of this script are written to hms_YYYY_fccscdl_grass.csv, hms_YYYY_fccscdl_cdlcrops.csv, hms_YYYY_fccscdl_wildland.csv, hms_YYYY_fccscdl_mwcrops.csv, and hms_YYYY_fccscdl_tree_crops.csv. Any changes to target year or input paths should be changed in both the hms_fccs.py and hms_canada_cropsplit.py scripts.

Those detects that occur on grasslands in the Flint Hills counties of Kansas during the annual burning season should be split from the rest of the grasslands for a separate processing stream described in in section 6. The split_fh.py helper script can be used to split the Flint Hills counties and annual date range into a separate file. More information on finding the annual date ranges is in section 6 of this document.

Two files containing HMS detects will be imported into SF2PY, one for Canada detects over wildlands and one for US detects over wildlands, including non-Flint Hills burn season grasslands. To create the US file the non-Flint Hills grasslands HMS file should be concatenated with the US wildland file. Example SF2PY input and Flint Hills HMS files from the 2023 EMBER run are included in this package for reference.

The overall order of scripts for preparing the HMS activity for SF2PY is gethms.py, gen_hms.csh/hms_set_fips.py, hms_canada_cropsplit.py, hms_fccs.py, and split_fh.py.

WFIGS US Wildland Perimeters

WFIGS US wildland fire perimeters (https://data-nifc.opendata.arcgis.com/datasets/nifc::wfigs-interagency-fire-perimeters/explore) are produced by the National Interagency Fire Center to collect active wildland fire geospatial data from across the US in one place. This dataset mainly contains wildfires but can also contain prescribed fires, particularly those on federal lands. The WFIGS data should be retrieved from the NIFC website as a shapefile so that the field names match those in the preprocessing script.

Specific variables need to be defined in the WFIGS shapefile so that it can be imported into SF2PY. These include the start date, end date, acres burned, fire name, fire type, and a unique fire identifier. Under the activity/wfigs path the setup_perimeters.csh script uses OGR to issue SQLite commands to define the variable names and the perimeters. The script assumes that it is run on the same day as the WFIGS retrieval. If it is not, then the table name (tbl) should be updated to reflect the proper shapefile and table name. An example WFIGS file is included in the package for reference.

NBAC Canada Wildland Perimeters

Like the WFIGS, the NBAC wildland fire perimeters (https://cwfis.cfs.nrcan.gc.ca/downloads/hotspots/) provide active fire perimeters. However, the NBAC product is produced from hotspots over Canada. Preparation for SF2PY is done using OGR to produce variables similar to those described in the WFIGS section. The get_perimeters.csh file under the activity/nbac folder will retrieve the active perimeters and perform the necessary changes to the shapefile. Perimeters from previous years are not considered active and may be retrieved from the archives (https://cwfis.cfs.nrcan.gc.ca/downloads/hotspots/archive/). An example Canada active perimeters shapefile is included in the package for reference.

# SF2PY Processing

Configuring SF2PY

SF2PY combines multiple input wildland fire activity data streams into a single activity stream for use in BSP. This section briefly covers where the configuration files are located and what modifications should be made for replicating EMBER for other years. More detailed information on the configuration of SF2PY is available for the EQUATES time series data set 6 and the 2020 NEI (https://www.epa.gov/system/files/documents/2023-03/NEI2020_TSD_Section7_Fires.pdf).

SF2PY requires an installation of Postgres with PostGIS. Python with the Pandas, Geopandas, and SQLAlchemy modules is required to run the scripts. SF2PY is known to work with Python 3.11.9, Pandas 2.1.4, Geopandas 0.14.2, and SQLAlchemy 1.4.51. Module version issues may be best remedied using a Conda or virtual environment matching those specific versions. The setup_tables.sh script under the src/sql path on the GitHub repository is used with smartfire_tables.sql to initialize the SF2PY database. Hostname (or address), username, and database name needs to be defined in the setup_tables.sh script and the pg.json file in src/config. The database password is specified in pg.json.

Data source configuration files are used to set location, reconciliation, clumping, and association variables for each dataset imported into SF2PY. For the 2023 EMBER SF2PY run four data sources were imported, which are defined in the four JSON files contained in the sf2/datasources path. The filename variable under the input section should be updated to match the system path where the input is located. Ancillary shapefiles are specified under the clumping and firetype sections of the two HMS configuration files. The paths to the shapefiles (fire_area_shapefile and shapefile) should be updated to match the location of each respective shapefile. If a new year or scenario is run the name parameter should be updated to reflect the change. Reusing a data source name will result in overwriting of the old data source.

Stream configuration files are used to set which data sources are reconciled into a single output stream and for which dates. The 2023 EMBER stream configuration file is located under sf2/streams. The export_path should be updated to point to an existing system path where SF2PY has write privileges. Any new stream year or scenario requires updates to the name, start_date, end_date, sources, and (optionally) export_path. As with data sources the use of the same name in a new SF2PY stream will result in a data overwrite.

Running SF2PY

SmartFire is run in two general phases: the data import phase and the reconciliation phase. All datasets used for reconciliation need to be completely imported before reconciliation is started. Some datasets may take many hours to a day to import. Two control scripts are included on the SF2PY github page: load_source.py and reconcile_fires.py

The load_source.py script loads a data source into SF2PY as defined in the JSON configuration. This needs to be called for every data source used in reconciliation. A progress counter will appear in interactive mode which indicates how far along the process is in each step. The JSON configuration is specified as a command line argument to the script. For example, to load the WFIGS data set the command line is: ./load_source.py wfigs_2023_nrt.json

The reconcile_fires.py script operates with a command line option pointing to the reconciliation stream configuration file. The reconciliation process generally takes a few hours depending on the number of data sources included. A progress counter will also show up in interactive mode. When the reconciliation is complete the entire scenario will be written to the export path.

SF2PY Outputs

The SF2PY single stream output files are written to the export path specified in the stream configuration file. Two sets of comma-delimited files are written for each day: events and fire_locations. The events file describes information of the name, size, date range, and data sources reconciled into each event. These files are not used directly by BSP. The fire_locations files contain the daily area burned and location for each fire. The fire_locations files are the primary activity data source for BSP.

Reference outputs from the 2023 EMBER run are provided in the sf2/summaries path.

# Preparing BSP Inputs

SF2PY fire_location file outputs are missing key data fields for use in the BSP. A set of scripts are applied to add time zone offset, county or province FIPS code, and split the activity into wild (WF) and prescribed (RX) fire types. A batch script, gen_summary.csh, is provided under the sf2/exports/summaries path to streamline the preparation process. This script should be placed in a subdirectory of the SF2PY output directory. The path to the SF2PY output directory should also be defined to for the workpath. Additional configuration is needed in the subscripts when setup on a new system.

The set_offset.py requires that the pytz, pandas, timezonefinder, zoneinfo, and GDAL (osgeo) python modules are installed. These can be installed through pip or conda. Under the “na_shape” class the shape_file variable should be updated to point to the path to the northamerica_county_province_state_latlon.shp file available in the geospatial path.

A new set of fire_location files will be written to the exports path. These files will be designated with “_utc_” in the file name and split into rx and wf fire types.

# BSP Processing

EMBER-Specific Configuration

BlueSky is provided on GitHub and includes installation instructions using multiple methods (https://github.com/pnwairfire/bluesky/blob/master/docs/installation.md). Modifications to components of BSP are necessary and therefor it is recommended to not use the Docker installation method. Version 4.2.14 of BSP was used for the 2023 EMBER runs. Later versions include updates to the emission factors that will produce inconsistencies with the 2023 EMBER emissions. If it is not possible to revert to this version of BSP, the emission factors used in the eflookup module for 2023 EMBER are included in the bsp/eflookup path. The emission factor tables may be placed in eflookup/fccs2ef/data of the system BSP installation to revert to this older version.

The EMBER platform uses custom sets of landcover rasters, FCCS over the US and FBP mapped to FCCS over Canada. These rasters are available in the geospatial folder of the package. EMBER also uses updated fuel loading cross-references in the BSP consumption module to match the landcover and FCCS versions. The updated fuel loading cross-reference file is available in the package at bsp/apps_consume/lf20_fccs4.csv. This cross-reference should replace the file under the BSP apps_consume module located at consume/input_data/fccs_loadings.csv in your system BSP installation.

Run configuration JSON files are in the bsp path of the package. There are two for the US (rx and wf) and one for Canada (wf_canada_mid). In each the location to the “fccs_fuelload_file” should be updated to match the location of the fuel rasters provided in the geospatial directory. The “dest_dir” path under “extrafiles” should also be updated to an existing output path with write privileges.

More information on configuring BlueSky runs is available in the GitHub documentation on configuration (https://github.com/pnwairfire/bluesky/blob/master/docs/configuration.md).

Running BSP

Two control scripts, run_bsp.csh and run_bsp.canada_mid.csh, are provided to run BSP for the US and Canada, respectively. The fire_locations files generated in step 4 of this document should be symbolically linked into the input directory as inputs to the BSP runs. A json directory, a logs directory, and the “extrafiles” directories defined in section 5.1 of this document should also be created and made writable for the BSP user. At the top of each script the workpath should be updated to match the path where this script is run.  Any lines related to “source” on specific shell scripts may be removed or replaced as needed for your BSP configuration. These scripts are setup to run 2023 for the months of April through October. Adjustments may be necessary for different years and scenarios.

More information on running BlueSky is available in the GitHub documentation (https://github.com/pnwairfire/bluesky/blob/master/docs/usage.md).

Generating FF10 Inventories

When complete BSP will have produced daily fire_location wildfire (wf) files for Canada and daily prescribed (rx) and (wf) files for the US. These can be found in the “extrafiles” path specified in each configuration file. To process these files into FF10 inventories the daily files should be concatenated and reformatted using the concat_fires.py script. The date range of the daily files to include in the output inventories should be specified at the top of the file with the startDay and endDay variables. The wrapper script concat_fires.csh is used to define the input path (location of the BSP output “extrafiles”) and the output filename for each run. All US fires (RX and WF) should be included in one file and all Canada fires (WF) should be in another.

A simple duplicate checker and record reduction script is found at bsp/dupes/dupes.py. The concatenated file is the first command line argument and a duplicates-removed file name is the second. For example, ./dupes.py ../sf2_2023_ember_us.csv sf2_2023_ember_us_nodupes.csv

Hazardous air pollutants (HAPs) emission factors are then added using the script bsp/haps/add_haps.py and add_haps_canada.py (for Canada). The arguments for this script are the duplicates-removed file and the output file with HAPs: ./add_haps.py ../dupes/sf2_2023_ember_us_nodupes.csv sf2_2023_ember_us_nodupes_haps.csv

The gen_fire_ff10.py script takes the concatenated daily data with HAPs and converts those files into a set of annual and daily FF10s for use in SMOKE. The wrapper script gen_ptfire_ff10.csh is used to call the script and specify the path to the concatenated file with HAPs, an inventory output label, and a work directory. Output annual FF10s will have all fields quoted. It is necessary to remove sets of empty double quotes from the annual FF10 files before running through SMOKE. This can be done with regular expressions using a tool such as sed. For the US the prescribed fire SCCs should be split for the wildland fire SCCs so that they can receive different profiles and sectors in SMOKE.

# Flint Hills

Prescribed burning of grasslands in the Flint Hills of Kansas and Oklahoma occurs every spring to reduce invasive species and improve graze for livestock. These fires can burn millions of acres per season and produce substantial smoke that impacts surrounding regions. The Flint Hills FF10s are created through a separate processing stream to use burned area supplied by Kansas State for scaling of HMS detects. Annual reports on burning in the Flint Hills are available from Kansas Flint Hills Smoke management (https://www.ksfire.org/).

In the HMS preparation steps (section 2.1) the Flint Hills were split from the other grasslands for specific counties and a year-specific date range. For 2024 the county list and date range is available in the KS Fire summary document: https://www.ksfire.org/docs/Flint_Hills_2024_Summary.pdf

This date information should be used in the split_fh.py for a future 2024 run. Information on acres-burned in each county are found in the fh/ancillary/fh_2016_2024.csv file of the package and goes through 2024.

The fh/flinthills_calc.py script is run to create the annual Flint Hills FF10s. A year-specific Flint Hills burn date range is set in the fh_dates variable, the HMS Flint Hills grassland input is set for the hms_fn variable, and a prefix label for the inventories are set with the prefix variable. All other files should remain the same. The output will be an annual and daily inventory to be run with the prescribed fires in SMOKE.

# SMOKE Processing

Ancillary Files

The SMOKE ancillary files used for the EMBER fire processing are based on the 2020 EPA Emissions Modeling Platform package (https://www.epa.gov/air-emissions-modeling/2020-emissions-modeling-platform). Most files can be found in the packages contained on the EPA FTP site (https://gaftp.epa.gov/Air/emismod/2020/ancillary_data/). Some files, including the NHAP exclude file, will need to be generated based on the fires that have VOC but do not have HAPs. A simple set of command line calls can be used to parse out the fires needed for the NHAP exclude from the smkinven log (smoke/nhapexclude.csh).

Run Configuration

The SMOKE run scripts are provided in three sets of two scripts: annual and daily scripts for the Canada, US RX, and US WF inventories. These scripts are found in the smoke directory of the package. All scripts should be updated so that the paths for the ancillary and input files point to the local system locations. For each inventory set the annual script should be run to completion. There is a known error in some versions of SMOKE with smkreport that causes a crash. This error can be safely ignored. Once the annual script has been run the daily scripts can be run to create the daily stack groups and inline files for each sector. Normal QA procedures such as inventory and post-SMOKE comparisons should be followed to ensure that sources are not dropped or duplicated.

References

(1) Simon, H.; Beidler, J.; Baker, K. R.; Henderson, B. H.; Fox, L.; Misenis, C.; Campbell, P.; Vukovich, J.; Possiel, N.; Eyth, A. Expedited modeling of burn events results (EMBER): A screening-level dataset of 2023 ozone fire impacts in the US. Data in Brief 2025, 58, 111208. DOI: https://doi.org/10.1016/j.dib.2024.111208.

(2) Larkin, N. K.; O'Neill, S. M.; Solomon, R.; Raffuse, S.; Strand, T.; Sullivan, D. C.; Krull, C.; Rorig, M.; Peterson, J.; Ferguson, S. A. The BlueSky smoke modeling framework. Int J Wildland Fire 2009, 18 (8), 906-920. DOI: https://doi.org/10.1071/WF07086.

(3) Raffuse, S. M.; Pryden, D. A.; Sullivan, D. C.; Larkin, N. K.; Strand, T.; Solomon, R. SMARTFIRE algorithm description. US Environmental Protection Agency, Research Triangle Park, NC, by Sonoma Technology, Inc., Petaluma, CA, and the US Forest Service, AirFire Team, Pacific Northwest Research Laboratory, Seattle, WA STI-905517-3719 2009.

(4) Raffuse, S. M.; Larkin, N. K.; Lahm, P. W.; Du, Y. Development of version 2 of the wildland fire portion of the National Emissions Inventory. In 20th International Emission Inventory Conference, Tampa, Florida, 2012.

(5) McNamara, D.; Stephens, G.; Ruminski, M.; Kasheta, T. The Hazard Mapping System (HMS)–NOAA multi-sensor fire and smoke detection program using environmental satellites. In 13th conference on satellite meteorology and oceanography, 2004; Vol. 22.

(6) Beidler, J.; Pouliot, G.; Foley, K. 2004-2017 Geospatial Dataset of Wild and Prescribed Fire Activity Over the Conterminous United States. Data in Brief 2024, 110856. DOI: https://doi.org/10.1016/j.dib.2024.110856.